SYNCHRONOUS FIFO
SVA-BASED VERIFICATION

Simulation, Assertions, Functional Coverage and Waveform Results

| Design | 8-bit synchronous FIFO |
| --- | --- |
| FIFO depth | 8 entries |
| Verification | SystemVerilog Assertions + SystemVerilog testbench |
| Simulation | Verilator 5.032 with VCD tracing |
| Waveform | GTKWave — waves/fifo.vcd |

Verification status: SUCCESSFUL

# 1. Objective

The objective is to verify an 8-bit, depth-8 synchronous FIFO using a SystemVerilog testbench, SystemVerilog Assertions (SVA), functional coverage, and waveform analysis. The completed environment exercises reset, normal FIFO operation, full/empty boundaries, data ordering, simultaneous read/write, and illegal boundary operations.

# 2. Verification Environment

The testbench instantiates the synchronous FIFO and a functional-coverage block. The captured testbench configuration uses DATA_WIDTH = 8, DEPTH = 8 and a count width derived from $clog2(DEPTH + 1). It also generates waves/fifo.vcd for waveform inspection.

DUT: sync_fifo

Testbench: fifo_tb

Assertions: fifo_sva.sv

Functional coverage: fifo_coverage.sv

Clock: 10 ns period

Waveform: waves/fifo.vcd

Simulator: Verilator 5.032

Waveform viewer: GTKWave

# 3. Test Plan and Results

| Test | Stimulus | Expected result | Status |
| --- | --- | --- | --- |
| 1. Reset | Apply and release reset | EMPTY, not FULL, count=0 | PASS |
| 2. Write 8 values | Write values 1 through 8 | FIFO reaches depth 8 | PASS |
| 3. FIFO FULL | Check full condition | full=1, count=8 | PASS |
| 4. Read 8 values | Read all entries | FIFO data order verified | PASS |
| 5. FIFO EMPTY | After all reads | empty=1, count=0 | PASS |
| 6. Write after EMPTY | Write 0xA5 | count=1 | PASS |
| 7. Read after write | Read 0xA5 | Returned data=0xA5 | PASS |
| 8. Corner cases | WRITE/FULL, RD+WR, drain, READ/EMPTY | Occupancy remains legal | PASS |
| 9. Final state | Final FIFO check | EMPTY, not FULL, count=0 | PASS |

The testbench source explicitly contains the nine verification stages above, including data-order checks and coverage-directed corner cases.

# 4. SVA Verification

Assertions were enabled during simulation. The verification checks the legal FIFO occupancy range and boundary behavior, including the protection of the FULL and EMPTY states. No assertion failure was reported in the completed simulation output.

FIFO occupancy remains within the legal range.

FIFO does not overflow beyond DEPTH.

WRITE while FULL does not increase occupancy.

READ while EMPTY does not corrupt occupancy.

Reset establishes the EMPTY state.

The final state returns to EMPTY.

# 5. Functional Coverage Results

| Coverage item | Observed | Meaning |
| --- | --- | --- |
| Writes | 19 | Write operations |
| Reads | 19 | Read operations |
| Simultaneous RD + WR | 1 | Both operations in one cycle |
| EMPTY | 10 | EMPTY state samples |
| FULL | 5 | FULL state samples |
| PARTIAL | 60 | Intermediate occupancy |
| COUNT = 0 | 10 | Zero occupancy |
| COUNT = 1 | 10 | One entry |
| COUNT = MID | 40 | Intermediate count |
| COUNT = DEPTH-1 | 10 | One below full |
| COUNT = DEPTH | 5 | Full occupancy |
| WRITE while FULL | 1 | Boundary operation |
| READ while EMPTY | 1 | Boundary operation |
| EMPTY -> NONEMPTY | 3 | State transition |
| NONEMPTY -> EMPTY | 3 | State transition |
| NONFULL -> FULL | 2 | State transition |
| FULL -> NONFULL | 2 | State transition |
| Total sampled cycles | 75 | Coverage sampling |
| Reset cycles | 1 | Reset activity |

# 6. Waveform Evidence

GTKWave successfully opened waves/fifo.vcd. The waveform shows clock, reset, FIFO count, EMPTY/FULL flags, read/write enables and read/write data. The displayed simulation reaches approximately 756 ns.

Figure 1. GTKWave waveform generated from waves/fifo.vcd.

# 7. Functional Coverage Evidence

Figure 2. Terminal output showing the completed FIFO functional coverage report.

# 8. Result Discussion

The completed run demonstrates successful execution of the FIFO test sequence and generation of both functional-coverage data and a VCD waveform. The coverage report shows that normal operation, EMPTY/FULL/PARTIAL states, occupancy boundaries, state transitions, simultaneous read/write, WRITE while FULL and READ while EMPTY were all exercised.

The corner-case test sequence intentionally attempts illegal boundary operations and then checks that FIFO occupancy remains unchanged. This provides direct verification of the boundary behavior rather than relying only on normal-operation tests.

# 9. Conclusion

The synchronous FIFO verification environment completed successfully. The final evidence consists of the completed test sequence, SVA-enabled simulation, functional coverage report, and GTKWave waveform. The verification covers the principal FIFO operating states and the specified boundary cases.

# 10. Reproduction Commands

verilator --binary --timing --trace --assert --top-module fifo_tb -Wno-WIDTH \

rtl/sync_fifo.sv assertions/fifo_sva.sv coverage/fifo_coverage.sv tb/fifo_tb.sv

./obj_dir/Vfifo_tb

gtkwave waves/fifo.vcd

# 11. Configuration Summary

DATA_WIDTH = 8 bits

DEPTH = 8 entries

COUNT_WIDTH = $clog2(DEPTH + 1)

Clock period = 10 ns

Top testbench = fifo_tb

VCD file = waves/fifo.vcd